SCHOOL OF MECHANICAL ENGINEERING

ASSIGNMENT-I

3RD SEM, MECHANICAL		           		DATE OF SUBMISSION: 22/08/10

GROUP-1

- What is thermodynamics? What does it predict? What is its area of application?

- Distinguish between the Macroscopic and Microscopic approach of thermodynamics?

- Discuss the concept of continuum?

- What are the characteristics of a Boundary of a thermodynamic system?

- What are thermodynamic properties of a system? Classify them? What is the condition for a characteristic to be a thermodynamic property?

- Discuss thermodynamic equilibrium?

- Derive temperature as a thermodynamic property from Zeroth law of thermodynamics?

- Discuss temperature scale in context to temperature measurement?

- Same numeric value is obtained in degree Centigrade and degree Fahrenheit scale for a particular bath. In Kelvin scale and degree Rankine scale what would be the value of this temperature.

- A pressure bottle stores 5 cubic metre of an inert gas at a pressure of 10 MPa and temperature 300K. Make calculations for the mass, density and specific volume of the gas. Molecular mass is 28.

GROUP-II

- If the specific gravity of 5 kg of fluid is 0.0012. Determine its volume.   [4.167 m3]

- One litre of gasoline weighs 7 N at a location where g is 9.81 m/s2. Determine the density of gasoline.							         [713.56 kg/m3]

- Two bodies, one with thermometer reading in Celsius and the other with thermometer reading in Kelvin, are in thermal equilibrium with a third system. What is the temperature of this system if both thermometers indicate the same value?   									  [ -136.58 0C]

- The lower half of a 6 m high cylinder tank is filled with water and the upper half with an oil of specific gravity 0.8. Determine the pressure at the bottom of the tank if the top is open to atmosphere.    					   [ 154.3 kPa]

- A vertical frictionless piston-cylinder device contains a gas at 620 kPa. The piston area is 50 cm2. Determine the piston mass, if the atmospheric pressure is 101 kPa.

[264.5 kg]

- The absolute pressure in water at a depth of 5 m is read to be 145 kPa. Determine (a) the local atmospheric pressure, and (b) the absolute pressure at a depth of 5 m in a liquid whose specific gravity is 0.85 at the same location.[96.0 kPa, 133.7 kPa]

- A vacuum gage connected to a tank reads 15 kPa at a location where the barometric reading is 750 mm Hg. Determine the absolute pressure in the tank. Take ρ of Hg 13,590 kg/m3.						   	      [85.0 kPa]

- The barometer of a mountain hiker reads 930 mbars at the beginning of a hiking trip and 780 mbars at the end. Neglecting the effect of altitude on local gravitational acceleration, determine the vertical distance climbed. Assume an average air density of 1.20 kg/m3.   				       	        [1274 m]

- The maximum blood pressure in the upper arm of a healthy person is about 120 mm Hg. If a vertical tube open to the atmosphere is connected to the vein in the arm of the person, determine how high the blood will rise in the tube. Take the density of the blood to be 1050 kg/m3.					         [1.55 m]

- Consider a U-tube whose arms are open to the atmosphere. Now water is poured into the U-tube from one arm, and light oil (ρ=790 kg/m3) from the other. One arm contains 70-cm-high water, while the other arm contains both fluids with an oil-to-water height ratio of 4. Determine the height of each fluid in that arm.

[0.168 m, 0.673 m]

- The pilot of an airplane reads the altitude 3000 m and the absolute pressure 58 kPa when flying over a city. Calculate the local atmospheric pressure in that city in kPa and in mm Hg. Take the densities of air and mercury to be 1.15 kg/m3 and 13,600 kg/m3, respectively.				   [91.8 kPa, 688 mm of Hg]

- Balloons are often filled with helium gas because it weighs only about one-seventh of what air weighs under identical conditions. The buoyancy force, which can be expressed as Fb = ρairgVballoon, will push the balloon upward. If the balloon has a diameter of 10 m and carries two people, 70 kg each, determine the acceleration of the balloon when it is first released. Assume the density of air is ρ=1.16 kg/m3, and neglect the weight of the ropes and the cage.

Also Determine the maximum amount of load, in kg, the balloon can carry.

[16.5 m/s2, 520.5 kg]

- A glass tube is attached to a water pipe. If the water pressure at the bottom of the tube is 115 kPa and the local atmospheric pressure is 92 kPa, determine how high the water will rise in the tube, in m. Take the density of water to be 1000 kg/m3.

[2.34 m]

- A new temperature scale in degree N is designed with ice point 100 degree N and steam point 400 degree N. What shall be the reading corresponding to 150 degree C? At what temperature both the Celsius and the new temperature scale reading would be same?

- Two spheres each of capacity 2 cum are connected by a pipe with a valve inserted in between. Sphere 1 contains Oxygen at 50 kPa and 320K. Sphere 2 contains Oxygen at 45 kPa and 290K. The valve is opened and the entire system is allowed to come to equilibrium. If temperature at this equilibrium is 300 K, determine final pressure neglecting volume of the pipe.		     [45.94kPa]

Faculty: T. R. Mahapatra

Asst. Prof. SME

9861425597

trupti_52@rediffmail.com